ROOF OVERHAUL OF THE IGAD ICPALD OFFICE & ASSOCIATED WORKS

|  | Description. | Unit | Quantity | Rate | USD |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I | BILL NO. 1 PRELIMINARIES Lighting and Power for the Works Contractors All Risk Insurance Performance Bond Transport Hoarding/Screening General Scaffolding Erecting, Maintaining and Dismantling of Sign Board Protection of Existing Property County approvals | 1 1 1 1 1 1 1 1 1 | SUM SUM SUM SUM SUM SUM SUM SUM SUM |  |  |
|  | PRELIMINARIES CARRIED TO MAIN SUMMARY |  |  |  |  |

|  | Description | Unit | Quantity | Rate | USD |
| --- | --- | --- | --- | --- | --- |
| A B C D E F | BILL NO. 2 |  |  |  |  |
|  | MAIN WORKS |  |  |  |  |
|  | DEMOLITIONS. |  |  |  |  |
|  | ROOF Carefully remove all tiles and underlay | SM | 610 |  |  |
|  | Carefully remove all timber trusses, fascia board, gutters | SM | 610 |  |  |
|  | CEILING. Demolish all the existing ceiling finishes and support frames | SM | 350 |  |  |
|  | ELECTRICAL INSTALLATIONS. Allow sum to carefully remove all lights from the ceilings and all electrical cables and storing them at a safe place. |  |  | SUM |  |
|  | PLUMBING INSTALLATIONS. Allow sum to carefully remove all water pipes in the ceiling, water tanks and any plumbing fittings that might be affected by the demolitions. |  |  | SUM |  |
|  | DISPOSAL Allow sum to Load and cart away all debris arising from ceiling and roof demolitions away from site. |  |  | SUM |  |
|  | DEMOLITIONS CARRIED TO MAIN SUMMARY |  |  |  |  |

|  | Description. | Unit | Quantity | Rate | USD |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K L M N O P | NEW ROOFING. Supply, fabricate and fix roof trusses and purlings in Medium gauge pre-galvanised LIGHT GAUGE STEEL(LGS) sections, joined by appropriate steel framing screws, driving sockets and align to the existing roof design and pitch. Supply and fix Heavy duty double-sided reflective roll with 5mm thick polythene foam core before fixing the roof covering for heat insulation. Supply and fix 26G coloured box-profiled roofing sheets to the new truss with appropriate roofing screws with rubber. Supply and fix 380mm wide 26G Flashing Supply and fix 225x25mm fascia and barge board in well celcured wrot cypress 150 x 100mm Boxed gutter including fixing with brackets at 1.00 metres centres in 18 SWG Galvanised MS sheet rainwater pipes with lapped and soldered joints in the running length, including coating joints with rust proof solution. 75mm dia Heavy gauge PVC Downpipe with MS 26G brackets every 1M 100mm dia heavy gauge PVC underground channel for rainwater FINISHES & FITTINGS. Supply and fix 75x25mm T&G in celcured wrot cypress fixed on and including LGS brandering to all eaves Supply and fix 12mm Gypsum ceiling complete with metal stud brandering Gypsum moulded cornice Skim, sand and apply 2 coats undercoat emulsion paint and 2 coats of First-grade vinyl matt emulsion paint to the ceiling surfaces. Apply 2 coats undercoat emulsion paint and 2 finishing coats to the eaves and fascia board Apply 2 coats of super gloss paint to downpipes and gutters ELECTRICAL INSTALLATIONS Allow sum to Fix lights and restore all removed electrical services PLUMBING INSTALLATIONS Allow sum to restore all dismantled plumbing connections and return removed water tanks to the underroof space. | SM SM SM LM LM LM LM LM SM SM LM SM SM LM | 610 610 610 16 120 95 20 45 95 350 290 350 149 115 | SUM SUM |  |
|  | NEW WORKS CARRIED TO MAIN SUMMARY |  |  |  |  |

|  |  |  |  | USD |
| --- | --- | --- | --- | --- |
| 1 2 3 | PROPOSED ROOFING MAKEOVER WORKS FOR IGAD ICPALD MAIN SUMMARY PRELIMINARIES DEMOLITIONS NEW WORKS |  |  |  |
|  | TOTAL COST OF WORKS |  |  |  |
| NOTES: 1 All works herein are executable within 6 weeks (1.5 months) MAIN SUMMARY |  |  |  |  |